| - The array name b is like a pointer: b == &b[0] - *b == *&b[0] == b[0] - Subtraction: pointer - pointer is the number of values between the addresses. |  |
| --- | --- |

| Precedences - NOT - AND/NAND - OR/XOR - IMPL - IFF |  |
| --- | --- |

| Setting a bit Use the bitwise OR operator (¦) to set a bit. number ¦= 1 << x; That will set bit x. | Clearing a bit Use the bitwise AND operator (&) to clear a bit. number &= ~(1 << x); That will clear bit x. | Toggling a bit The XOR operator (^) can be used to toggle a bit. number ^= 1 << x; That will toggle bit x. |
| --- | --- | --- |
| Checking a bit To check a bit, shift the number x to the right, then bitwise AND it: bit = (number >> x) & 1; | Changing the (n)th bit to x Setting the nth bit to either 1 or 0 can be achieved with the following: number ^= (-x ^ number) & (1 << n); | Mask with Runs of (1-0-1) 0…0000 1 000 = 1 << 3 0…0000 0111 = (1 << 3) - 1 0…000 111 00 = ((1 << 3) - 1) << 2 1…111 000 11 = ~(((1 << 3) - 1) << 2) ~(((1 << 3) - 1) << 2) |

Von Neumann Architecture

| (a) What are the three main parts of a von Neumann computer? CPU, Memory, and I/O devices (b) What is unique about Von Neumann architecture? Programs are stored as data in memory. (c) What are the parts of the CPU? Control unit and (Arithmetical/Logical) Processing Unit (a) What is the Memory Address Register? The (MAR) holds the location to read or write (b) What is the Memory Data Register? It holds the value read from memory or the value to write to memory. (c) What are the steps involved in reading and writing memory? To read: Set MAR to the address to read; signal Read; Find the value in the MDR and copy it out to wherever. To write: Set (MAR) to the address to read; Set MDR ← value to write; Signal Write What does the Program Counter point to? Why is it badly named? The PC points to the next instruction to execute. What are the three basic types of instructions? Calculation instructions use and create values; Data movement instructions move data to/from memory; Control instructions perform goto operations. | (a) What are the phases of the instruction cycle? Fetch Instruction, Decode Instruction, Evaluate Addresses, Fetch Operands, Execute Instruction, Store Results. What are the steps of the Fetch Instruction phase? Read the instruction pointed to the program counter from memory into the instruction register; then increment the program counter During the Decode Instruction phase of the instruction cycle, where is the instruction being decoded? The instruction is in the instruction register. What happens during the Evaluate Addresses (a.k.a Get Effective Addresses) phase of the instruction cycle? Why is this phase sometimes skipped? We figure out where the operands are stored; this could be a register number or a memory address. What happens during the Fetch Operands phase of the instruction cycle? From where can operands be fetched? During Fetch Operands we actually retrieve the operand values from a register, from memory, or from part of the instruction register. (f) What happens during the Store Results phase of the instruction cycle? To where can results be stored? Why is the phase sometimes skipped? During Store Results, values calculated during Execute Instruction are moved to memory or CPU registers. This phase is skipped if there are no instructions. |
| --- | --- |